Add a data integrity requirement

Here is the updated specification with the data integrity section added:

6. Data Integrity Requirements

6.1 Write-Through Integrity

REQ-DI-1 (No Overwrite): Data written to a FIFO location shall not be overwritten until the read pointer has advanced past that location. The write pointer shall never equal the read pointer at the same memory index while the FIFO is not empty; that is, the write binary count lower bits shall never equal the read binary count lower bits unless wfull=1 (wrap condition) or the FIFO has been fully drained (rempty=1).

REQ-DI-2 (Write Capture): Data presented on wdata shall be captured into FIFO memory on the rising edge of wclk at which the valid write condition holds (wen_n=0 AND wfull=0). Data on wdata that is presented when wen_n=1 or wfull=1 shall never be written to memory.

6.2 Read-Out Integrity

REQ-DI-3 (Read Stability): rdata shall reflect the memory contents at the current read pointer address at all times. Between two consecutive valid reads, rdata shall remain stable (the memory contents at that address do not change unless overwritten by the write domain — which REQ-DI-1 prohibits).

REQ-DI-4 (FIFO Order): Data shall be read out in the same order in which it was written. The Nth read shall return the data written by the Nth write, for all N ≥ 1. This first-in first-out ordering is enforced by the monotonically incrementing binary counters on both the write and read sides.

6.3 Pointer Consistency

REQ-DI-5 (Write Ahead of Read): The write binary count shall never be less than the read binary count when measured in the same clock domain and accounting for the wrap bit. That is, the number of unread entries (occupancy) shall always be in the range [0, DEPTH].

REQ-DI-6 (Occupancy Bound): At no point in time shall the number of valid (written but not yet read) entries exceed DEPTH. This is guaranteed by the wfull flag deasserting further writes when occupancy reaches DEPTH, and enforced by REQ-ENV-1.

6.4 Assertions for Data Integrity (FV)

The following assertion and cover requirements correspond directly to the requirements above:

| Requirement | Property Type | Label | What to Check |
| --- | --- | --- | --- |
| REQ-DI-1 | ap_ | ap_no_overwrite | !rempty → write binary count lower bits ≠ read binary count lower bits at same address |
| REQ-DI-2 | ap_ | ap_write_captured | Valid write → ##1 fifomem[past_waddr] == $past(wdata) |
| REQ-DI-3 | ap_ | ap_rdata_stable | No valid read AND no write to current rptr address → ##1 rdata == $past(rdata) |
| REQ-DI-4 | ap_ | ap_fifo_order | Sequential writes then reads return data in write order (BMC depth ≥ DEPTH cycles) |
| REQ-DI-5 | ap_ | ap_occupancy_nonneg | wrbincount_wrside[DEPTHEXP] XOR rdbincount_rdside[DEPTHEXP] encodes non-negative occupancy |
| REQ-DI-6 | ap_ | ap_occupancy_bounded | Occupancy ≤ DEPTH at all times; wfull=1 prevents further writes |
| REQ-DI-1 | cp_ | cp_no_overwrite_hit | Cover: write and read pointers approach same lower-bit address without collision |
| REQ-DI-4 | cp_ | cp_fifo_order_hit | Cover: at least 2 writes followed by 2 reads completing in order |
| REQ-DI-6 | cp_ | cp_occupancy_at_max | Cover: occupancy reaches exactly DEPTH |

6.5 Ambiguity Gate — RQM0

⚠️ REQ-DI-2 (ap_write_captured) and REQ-DI-4 (ap_fifo_order) reference internal memory array fifomem. Formal tools require explicit internal observability ports or bind-file access to fifomem. Clarification required before generating SVA: Should fifomem be exposed as an output port in a dedicated FPV wrapper, or accessed via hierarchical reference in the bind module?

Step 7 gate active — waiting for your selection A/B/C/D/E. No SVA code will be generated until you reply.

- A — Write ALL assertions + assumptions + cover for data integrity section

- B — Write assertions only

- C — Write cover points only

- D — Generate FPV wrapper with fifomem exposed, then write SVA

- E — Skip code — specification text only