write a formal English requirement specifications of this fifo.

This specification should be clear so tath AI can covert it to RTL and assertions

Here is the formal English requirement specification for fifo_async:

Formal Requirement Specification: Asynchronous FIFO (fifo_async)

1. Overview

The fifo_async module is a dual-clock, first-in first-out (FIFO) memory buffer that transfers data between two independent clock domains. The write port operates on wclk and the read port operates on rclk. Pointer synchronization between domains is achieved using Gray-code encoding and two-stage flip-flop synchronizers to prevent metastability. The FIFO depth is 2^DEPTHEXP entries, and the data width is WIDTH_G bits, both parameterized at instantiation.

2. Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| WIDTH_G | integer | 8 | Data bus width in bits |
| DEPTHEXP | integer | 2 | Depth exponent; FIFO depth = 2^DEPTHEXP entries |

3. Port Definitions

| Port | Direction | Domain | Polarity | Description |
| --- | --- | --- | --- | --- |
| wclk | input | write | — | Write-domain clock |
| wrst_n | input | write | active-LOW | Asynchronous write-domain reset |
| wen_n | input | write | active-LOW | Write enable; a write occurs when wen_n=0 and wfull=0 |
| wdata[WIDTH_G-1:0] | input | write | active-HIGH | Data to be written into the FIFO |
| wfull | output | write | active-HIGH | Asserted when the FIFO is full; no write may occur |
| rclk | input | read | — | Read-domain clock |
| rrst_n | input | read | active-LOW | Asynchronous read-domain reset |
| ren_n | input | read | active-LOW | Read enable; a read occurs when ren_n=0 and rempty=0 |
| rdata[WIDTH_G-1:0] | output | read | active-HIGH | Data read from the FIFO |
| rempty | output | read | active-HIGH | Asserted when the FIFO is empty; no read may occur |

4. Functional Requirements

4.1 Reset Behavior

REQ-RST-1 (Write Reset): When wrst_n is asserted low (asynchronously), the write pointer and write binary count shall be cleared to zero on the next rising edge of wclk. The wfull flag shall be deasserted (wfull=0) after reset.

REQ-RST-2 (Read Reset): When rrst_n is asserted low (asynchronously), the read pointer and read binary count shall be cleared to zero on the next rising edge of rclk. The rempty flag shall be asserted (rempty=1) after reset, indicating the FIFO is empty.

REQ-RST-3 (Synchronizer Reset): Both 2-FF synchronizer stages (write-pointer-to-read-domain and read-pointer-to-write-domain) shall be cleared to zero when their respective domain reset is asserted.

4.2 Write Operation

REQ-WR-1 (Write Condition): A write shall occur on the rising edge of wclk if and only if wen_n=0 AND wfull=0.

REQ-WR-2 (Write Data): On a valid write, wdata shall be stored into the memory location indexed by the lower DEPTHEXP bits of the write binary count at the time of the write.

REQ-WR-3 (Write Pointer Advance): On a valid write, the write binary count shall increment by 1, and the write Gray-code pointer waddr_wrside shall be updated to the Gray encoding of the new binary count.

REQ-WR-4 (Write Pointer Stable): When no valid write occurs (wen_n=1 or wfull=1), the write binary count and write Gray-code pointer shall not change.

REQ-WR-5 (No Overflow): The write binary count shall never exceed 2×DEPTH−1. The environment (master) must not assert wen_n=0 while wfull=1.

4.3 Read Operation

REQ-RD-1 (Read Condition): A read shall occur on the rising edge of rclk if and only if ren_n=0 AND rempty=0.

REQ-RD-2 (Read Data): rdata shall continuously reflect the FIFO memory location indexed by the lower DEPTHEXP bits of the current read binary count (combinational, not registered).

REQ-RD-3 (Read Pointer Advance): On a valid read, the read binary count shall increment by 1, and the read Gray-code pointer raddr_rdside shall be updated to the Gray encoding of the new binary count.

REQ-RD-4 (Read Pointer Stable): When no valid read occurs (ren_n=1 or rempty=1), the read binary count and read Gray-code pointer shall not change.

REQ-RD-5 (No Underflow): The read binary count shall never exceed 2×DEPTH−1. The environment must not assert ren_n=0 while rempty=1.

4.4 Gray-Code Pointer Encoding

REQ-GRAY-1: The write Gray-code pointer waddr_wrside shall at all times equal the Gray encoding of wrbincount_wrside, defined as wrbincount_wrside XOR (wrbincount_wrside >> 1).

REQ-GRAY-2: The read Gray-code pointer raddr_rdside shall at all times equal the Gray encoding of rdbincount_rdside, defined as rdbincount_rdside XOR (rdbincount_rdside >> 1).

REQ-GRAY-3: On each pointer increment, exactly one bit of the Gray-code pointer shall change (single-bit transition property of Gray code).

4.5 Pointer Synchronization (CDC)

REQ-SYNC-1 (Write-to-Read): The write Gray-code pointer waddr_wrside shall be synchronized into the read clock domain through a two-stage flip-flop chain: stage-1 register waddr_rdside0 captures waddr_wrside on every rising rclk, and stage-2 register waddr_rdside captures waddr_rdside0 on the following rclk cycle.

REQ-SYNC-2 (Read-to-Write): The read Gray-code pointer raddr_rdside shall be synchronized into the write clock domain through a two-stage flip-flop chain: stage-1 register raddr_wrside0 captures raddr_rdside on every rising wclk, and stage-2 register raddr_wrside captures raddr_wrside0 on the following wclk cycle.

REQ-SYNC-3 (Metastability): Gray-code encoding is mandatory to ensure that only one bit changes per pointer increment, minimizing the risk of multi-bit metastability during synchronization.

4.6 Full Flag Generation (Write Domain)

REQ-FULL-1: The wfull flag shall be generated in the write clock domain. The synchronized read pointer raddr_wrside shall be decoded from Gray to binary to produce rdbincount_wrside.

REQ-FULL-2: wfull shall be asserted (wfull=1) when and only when: the lower DEPTHEXP bits of rdbincount_wrside equal the lower DEPTHEXP bits of wrbincount_wrside, AND the MSB of rdbincount_wrside differs from the MSB of wrbincount_wrside.

REQ-FULL-3: wfull shall be deasserted (wfull=0) in all other conditions.

4.7 Empty Flag Generation (Read Domain)

REQ-EMPTY-1: The rempty flag shall be generated in the read clock domain using the synchronized write pointer.

REQ-EMPTY-2: rempty shall be asserted (rempty=1) when and only when the read Gray-code pointer raddr_rdside equals the synchronized write Gray-code pointer waddr_rdside.

REQ-EMPTY-3: rempty shall be deasserted (rempty=0) when the two pointers differ.

4.8 Environment Constraints (Assumptions for FV)

REQ-ENV-1: The write master shall never assert wen_n=0 (write enable) while wfull=1.

REQ-ENV-2: The read master shall never assert ren_n=0 (read enable) while rempty=1.

REQ-ENV-3 (Fairness): Over any unbounded time interval, the write master shall eventually assert wen_n=0 at least once, and shall eventually deassert it (wen_n=1) at least once. This prevents livelock in formal verification.

REQ-ENV-4 (Fairness): Over any unbounded time interval, the read master shall eventually assert ren_n=0 at least once, and shall eventually deassert it (ren_n=1) at least once.

5. Assertion Coverage Requirements

Every functional requirement above shall have at least one cover property (cp_) demonstrating that the scenario described is reachable by the formal tool. Every assumption (mp_) shall have a corresponding cp_hit cover point per UR-5.